🌐 How to Link the Static Library (.lib) to a Keil MDK 5 Project

The LCD panel static graphic libraries (.lib) are located within the KVB_LCD project folder, alongside the GUI and evaluation files (.c and .h).

📂 File Structure Overview

- KVB_LCD.h – Main GUI (Graphical User Interface) header file.

- KVB_TEST.h – Header file containing two evaluation/presentation functions tailored for different screen resolutions.

- KVB_TEST.c – Source file implementing the two resolution-dependent evaluation functions.

- KVB_TEXT.h – Header file dedicated to the text variables used in the evaluation code.

- KVB_TEXT.c – Source file containing the text variable definitions for the evaluation code.

- LCD_16bit_Open407_F407VGT6_9325.lib – Static library file for the ILI9325 LCD panel (320 x 240 resolution).

- LCD_16bit_Open407_F407VGT6_9341.lib – Static library file for the ILI9341 LCD panel (320 x 240 resolution).

- LCD_16bit_Open407_F407VGT6_9481.lib – Static library file for the ILI9481 LCD panel (480 x 320 resolution).

- LCD_16bit_Open407_F407VGT6_9486.lib – Static library file for the ILI9486 LCD panel (480 x 320 resolution).

- LCD_16bit_Open407_F407VGT6_8009A.lib – Static library file for the OTM8009A LCD panel (800 x 480 resolution).

⚙️ Linker and GUI Configuration Steps

- In the left panel of Keil (Project Tree), expand the KVB_LCD folder. If a .lib library for another LCD panel is already present, remove it: right-click the .lib file and select Remove File 'xxxx.lib' from the context menu. Note: Only one graphic library file must be linked in the project at a time.

- Right-click the KVB_LCD folder in the Project Tree and select Add Existing Files to Group...

- In the file selection window, click the Files of type drop-down menu at the bottom and switch it to Library file (*.lib; *.a).

- Select your specific target library file (e.g., LCD_16bit_Open407_F407VGT6_xxxx.lib) and click Add.

- Keil MDK is now configured to link this pre-compiled binary file with your main code.

- Open the main GUI header file KVB_LCD.h, locate line 120, and uncomment strictly the line corresponding to your target LCD panel. This line contains the averaged timing configuration for the panel's FSMC bus. These values depend heavily on the specific hardware implementation of the panel, wire lengths, and ribbon cable traits.

⚠️ CRITICAL HARDWARE NOTE: Due to the extremely high data transfer rates into the display's GRAM, if you are using a flat ribbon cable, it is MANDATORY to install a 22 pF signal reflection dampening capacitor at the very end of the ribbon cable. Place it as close as physically possible to the panel's interface connector on the WE (Write Enable) signal line.

- Finally, navigate to your project's main.c file and uncomment one of the two evaluation functions depending on your display's resolution: lcd_Test480x320Landscape() or lcd_Test320x240Landscape().

During the compilation process, Keil MDK 5 will automatically fetch and link the optimized binary code from your selected library file (e.g., LCD_16bit_Open407_F407VGT6_9486.lib).